FusWorth

AC15-A

Product Specifications

| Specification | Details |
| --- | --- |
| Source Specification | Metal Card Reader |
| Main Unit Dimensions | 88*88*18mm |
| Back Cover Dimensions | 86*86*2mm |
| Standard | Complies with ISO/IEC 14443 Type A |
| Housing Material | Metal |
| Operating Voltage | 9-12VDC |
| Operating Current | 70mA |
| Protection Rating | Waterproof and Dustproof (IP65) |
| Supported Card | MIFARE One IC card |
| Keypad | Touch keypad with LED digits |
| Operating Frequency | IC (13.56MHz) |
| Operating Environment | Operating Temperature: -10~50℃ Operating Humidity: <90%< p=""> |
| Transmission Distance | 80m max. |
| Reading Distance | IC 3-5cm depending on card |
| Output Mode | WG26/34 |
| Pulse Output | Negative pulse widthTP=100μs, Pulse periodTW=2ms |